8TMDP H-D9Tu5rr8et 5NeStw3orBk Camera

CAMERA

Secure The World With You

H.265+/H.265/H.264+/H.264/MJPEG coding

Max.resolution: 8MP ( 3840 × 2160 ) ICR auto switch, true day / night vision

3D DNR, true WDR, HLC, BLC, smart IR and ROI coding 30~50m IR night view distance

1CH audio input; 1CH built-in MIC Built-in micro SD card slot, up to 256GB DC12V/PoE power supply

Support three streams IP 67

Support mobile surveillance by smart phones with iOS and Android OS Intelligent analytics

H.265

H.265

PoE

Motion Detection

WDR	AWB

PoE	IP67

Defog

VCA

HLC

-30°~60°

BLC

en.tvt.net.cn

For more information, please visit our website

Shenzhen TVT DIGITAL TECHNOLOGY CO., LTD.

Address: 23rd Floor, Block B4, Building NO.9, Shenzhen Bay Eco-Technology Park, Nanshan District, Shenzhen, Guangdong Province, P.R.China

Tel: +86-755-36995888	Fax: +86-755-33104777

Web site: http://en.tvt.net.cn	E-mail: overseas@tvt.net.cn

TD-9585S3B

Camera

8MP Turret Network

| Model Specifications | TD-9585S3B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Camera |  |  |  |  |  |
| Image Sensor | 1 / 2.8 "CMOS |  |  |  |  |
| Image Size | 3840×2160 |  |  |  |  |
| Electronic Shutter | 1 / 2 s ~ 1 / 100000 s |  |  |  |  |
| Iris Type | DC Iris |  |  |  |  |
| Min. Illumination | 0.02 lux @ F1.6, AGC ON; 0 lux with IR ~ |  |  |  |  |
| Lens | 2.8 12mm@F1.6, motorized ~ |  | ~ | ~ |  |
| Field of View | Horizontal:99° 33°; Vertical :53° 17°; Diagonal:117° 37° Φ |  |  |  |  |
| Lens Mount | 14 |  |  |  |  |
| Day&Night | ICR |  |  |  |  |
| Wide Dynamic Range | 120dB |  |  |  |  |
| BLC | Yes |  |  |  |  |
| HLC | Yes |  |  |  |  |
| Defog | Yes |  |  |  |  |
| Digital NR | 3D DNR |  |  |  |  |
| S/N Ratio | ≥50dB |  |  |  |  |
| Angle Adjustment | Pan: 0°~360°; Tilt:0°~80°; Rotation: 0°~360° |  |  |  |  |
| DORI Distance |  |  |  |  |  |
| Lens | Detect | Observe |  | Recognize | Identify |
| 2.8~12mm(W) | 86.65m(284.28ft) | 34.66m(113.71ft) |  | 17.33m(56.85ft) | 8.66m(28.41ft) |
| 2.8~12mm(T) | 289.06m(552.31ft) | 115.62m(240.12ft) |  | 57.81m(136.06ft) | 28.91m(84.04ft) |
| Image |  |  |  |  |  |
| Video Compression | Main stream: H.265+ / H.265 / H.264+ / H.264 Sub stream: H.265+ / H.265 / H.264+ / H.264 / MJPEG Third stream: H.265+ / H.265 / H.264+ / H.264 / MJPEG |  |  |  |  |
| H.264 Compression Standard | Baseline Profile/Main Profile/High Profile |  |  |  |  |
| Resolution | 8MP ( 3840 × 2160 ), 6MP( 3072 × 2048 ),5MP( 2592 × 1944), 4MP ( 2592 × 1520), 2K(2560 × 1440 ), 3MP(2304 × 1296), 1080P ( 1920 × 1080 ), 720P ( 1280 × 720 ), D1, CIF, 480 × 240 |  |  |  |  |
| Main Stream | 60Hz : 8MP(1~20fps)/6MP/5MP/4MP/2K/3MP/1080P/720P(1~30fps) 50Hz : 8MP(1~20fps)/6MP/5MP/4MP/2K/3MP/1080P/720P(1~25fps) |  |  |  |  |
| Sub Stream | 60Hz: 720P(1~12fps)/D1/480 × 240(1~30fps) 50Hz: 720P/D1/480 × 240(1~25fps) |  |  |  |  |
| Third Stream | 60Hz: 720P/D1/CIF/480 × 240 (1~30fps) 50Hz: 720P/D1/CIF/480 × 240 (1~25fps) |  |  |  |  |
| Bit Rate | 64 Kbps ~ 12 Mbps |  |  |  |  |
| Bit Rate Type | VBR / CBR |  |  |  |  |
| Audio Compression | G711A / U |  |  |  |  |
| Image Settings | ROI, Saturation, Brightness, Hue, Contrast, Wide Dynamic, image mirror, image flip, Sharpness, NR, etc. adjustable through client software or web browser |  |  |  |  |
| ROI | Each ROI to be configured separately |  |  |  |  |
| Interfaces |  |  |  |  |  |
| Network | RJ 45 10M/100M self-adaptive Ethernet port |  |  |  |  |
| Video Output | No |  |  |  |  |
| Audio | 1CH built-in MIC; 1CH audio input |  |  |  |  |
| Alarm | No |  |  |  |  |
| Storage | Built-in micro SD card slot; up to 256GB |  |  |  |  |
| Reset | Yes |  |  |  |  |
| Functions |  |  |  |  |  |
| Remote Monitoring | IE browser/ TVT NVR / NVMS / mobile APP |  |  |  |  |
| Web Browser | IE11(plug-in required)/Chrome 89.0+/Edge 89.0+/Firefox 87.0+/Safari 14.0+ |  |  |  |  |
| Online Connection | Support simultaneous monitoring for up to 10 users; Support multi-stream real time transmission |  |  |  |  |
| Network Protocol | UDP, IPv4, IPv6, DHCP, NTP, RTSP, RTP, RTMP, RTCP, PPPoE, DDNS, SMTP, FTP, SNMP, HTTP, HTTP POST, 802.1x, UPnP, HTTPs, QoS |  |  |  |  |
| Interface Protocol | ONVIF |  |  |  |  |
| Storage | Network remote storage; micro SD card storage |  |  |  |  |
| Smart Alarm | Motion detection; SD card error; SD card full |  |  |  |  |
| Intelligent Analytics | Scene change, video blur detection, abnormal color detection , line crossing detection, intrusion detection |  |  |  |  |

SPECIFICATIONS

| General function | Watermark, video mask, heartbeat,corridor pattern |
| --- | --- |
| Security | Video encryption, firmware encryption, HTTPs encryption, configuration encryption, illegal login lock, IP address filtering, 802.1x authentication, basic and digest authentication for HTTP/HTTPs |
| IR Distance | 30~50m |
| Protection Grade | IP 67 |
| PoE | Yes (IEEE802.3af) |
| Others |  |
| Power Supply | DC12V/PoE |
| Power Consumption | < 8W |
| Storage Conditions | - 30 °C ~ 65°C (-22°F~149°F), Humidity: less than 95 % (non-condensing) |
| Operating Conditions | - 30 °C ~ 60 °C (-22°F~140°F), Humidity: less than 95 % (non-condensing) Φ |
| Dimensions ( mm ) | 111.5 × 99.6 |
| Weight ( net ) | Approx. 0.661KG |
| Installation | Ceiling mounting; Wall mounting |
| Certificate | CE, FCC |
| Environmental Protection | Complies with Directive EU RoHS, WEEE(2012/19/EU) , directive 94/62/EC and REACH(EC1907/2006) |

111.50mm

99.60mm

111.50mm

DIMENSIONS

AVAILABLE ACCESSORIES

TD-YXH0302+ TD-YZJ0811

TD-YXH0302+

TD-YZJ0407+TD-YZJ0601

TD-YXH0302+

TD-YZJ0407+TD-YZJ0501

TD-YZJ0203

TD-YXH0106

TD-YXH0302

AVAILABLE MODEL

en.tvt.net.cn

*All pictures and specifications are for reference only and are subject to change without prior notice.

TD-9585S3B(D/AZ/PE/AR3)

Available Model